SYSON# aur SUSP# dono SIO (EC) ke important output signals hote hain. Inka kaam power sequence ke dauran alag-alag power rails ko ON karna hota hai.

SYSON# aur SUSP# dono Enable signals hain, lekin dono ka kaam alag hai.

# 1. SYSON# (System ON)

### Kya hai?

SYSON# = System ON Signal

Ye signal SIO se nikalta hai aur PWM/Buck Controller ko enable karta hai.

### Voltage

| State | Voltage |
| --- | --- |
| Laptop OFF | 0V |
| Laptop ON | 3.3V |

### Kaam

- Main power rails enable karna.

- CPU, RAM aur chipset ki power start karna. (CPU / RAM / PCH Power)

# 2. SUSP# (Suspend Signal)

### Kya hai?

SUSP# = Suspend Signal

Ye bhi SIO ka output hota hai.

Laptop Sleep (S3) aur Resume ke dauran ye signal power regulators ko control karta hai.

### Voltage

| State | Voltage |
| --- | --- |
| Suspend | 0V |
| Normal ON | 3.3V |

### Kaam

- Sleep Mode Control

- Resume Control

- Secondary Power Rails Enable

# SYSON# vs SUSP#

| Signal | Function |
| --- | --- |
| SYSON# | System Power ON |
| SUSP# | Suspend / Resume Control |

# Sleep Mode Me Kya Hota Hai?

# Laptop chal raha hai.

# Ab Sleep dabaya.

Dhyan do:

👉 Main power abhi bhi ON hai.

Sirf secondary devices band hue hain.

# Resume

Mouse hilaya.

# Voltage Table

| Laptop State | SYSON# | SUSP# |
| --- | --- | --- |
| OFF | 0V | 0V |
| ON | 3.3V | 3.3V |
| Sleep | 3.3V | 0V |
| Resume | 3.3V | 3.3V |

# Common SIO Output Signals in Different ODMs

| Function | Compal | Quanta | Wistron | Inventec | LCFC (Lenovo) |
| --- | --- | --- | --- | --- | --- |
| Main System ON | SYSON# | SUS_ON | MAINON | SYS_ON | EC_ON / S5_ENABLE# |
| Power Enable | EC_ON# | EC_ON | EC_ON | EC_ON | EC_ON |
| Suspend Control | SUSP# | SUSC# / SUS_ON | SUSP# | SUSP# | SUS_ON / SUSP# |
| Power Button to PCH | PM_PWRBTN# | PM_PWRBTN# | PM_PWRBTN# | PM_PWRBTN# | PM_PWRBTN# |
| Resume Reset | RSMRST# | RSMRST# | RSMRST# | RSMRST# | RSMRST# |