SLP_S4# aur SLP_S3# PCH (ya BayTrail SoC) ke output signals hote hain.

Ye signals EC (SIO) ko batate hain:

"Main Sleep State se bahar aa gaya hoon, ab tum remaining power rails ON kar sakte ho."

Uske baad EC apne output enable signals generate karta hai.

Different boards me in signals ke naam alag ho sakte hain:

- SYSON

- SUSON

- SUSP#

- SUS_PWR

- MAINON

- VR_ON

Ye sab Enable Signals hote hain, jinka kaam DC-DC Converter / VRM Controller ko ON karna hota hai, jisse system ki remaining voltages (1.05V, 1.8V, VCORE, DDR Voltage, etc.) generate ho sake.

# DC-DC Converter Kya Hota Hai?

DC-DC Converter ek electronic circuit ya IC hota hai jo ek DC Voltage ko doosri DC Voltage me convert karta hai.

Example

Input : 19V DC

↓

DC-DC Converter

↓

Output: 5V DC

Laptop Adapter sirf 19V deta hai.

Lekin motherboard ke alag-alag IC ko alag voltage chahiye.

| Device | Required Voltage |
| --- | --- |
| EC (SIO) | 3.3V |
| BIOS | 3.3V / 1.8V |
| RAM | 1.35V |
| PCH/SoC | 1.05V |
| CPU Core | 0.8V ~ 1.2V |

## DC-DC Converter

Ye poora power circuit hota hai.

Iske andar hote hain:

- PWM / VRM Controller IC

- MOSFET

- Coil (Inductor)

- Capacitor

- Feedback Circuit

# PWM IC Kya Hota Hai? - PWM = Pulse Width Modulation

PWM IC (Pulse Width Modulation Controller) ek Controller IC hota hai jo MOSFET ko ON/OFF karke required voltage generate karwata hai

Simple language me:

PWM IC = DC-DC Converter ka "Brain"

# PWM IC Kya Check Karta Hai?

Ye continuously monitor karta hai

- Input Voltage

- Output Voltage

- Current

- Temperature (kuch IC me)

- Enable Signal

VRM Controller vs PWM Controller

Har VRM Controller ek PWM Controller hota hai, lekin har PWM Controller VRM Controller nahi hota. Ye sabse sahi aur technical definition hai.

| Voltage | Controller Name |
| --- | --- |
| 3.3V | PWM Controller |
| 5V | PWM Controller |
| 1.8V | PWM Controller |
| 1.05V | PWM Controller |
| CPU VCORE | VRM Controller |
| GPU CORE | VRM Controller |

## Main SoC Core Voltage (Power ON ke baad)

Power button press karne ke baad:

PWRBTN#

↓

RSMRST#

↓

SUSCLK

↓

SLP_S4#

↓

SLP_S3#

↓

SYSON / SUSON

↓

PWM IC ON

↓

+1.05VS

PCH/SoC - 1.05V

Ye +1.05VS ya VCCCORE / VCCIO wali voltage hoti hai.

Iska kaam:

- SoC Internal Logic

- Memory Controller

- PCIe

- SATA

- USB

- Graphics Logic

Ye tabhi aati hai jab system ON hota hai.

Random

Supply for PCH 1.05VALW / VCCPRIM_CORE (Platform dependent)

## Simple Comparison

| Voltage | Kab aati hai? | Kaam |
| --- | --- | --- |
| +1.0VALW | Adapter lagte hi | Always ON / RTC / PMU |
| +1.05VS | Power Button ke baad | Main SoC Working |

  +1.0VALW → SoC ko "jagaye rakhti" hai (Always Domain).

  +1.05VS → SoC ka main processing section chalati hai.

+1.05V Ye voltage SoC ke main working logic ko chalati hai.

Bilkul waise hi:

- +1.0VALW = Always ON Power

- +1.05VS = Main Working Power

## Yaad rakhne ka shortcut

| Voltage | Source | Kaam |
| --- | --- | --- |
| RTC_VCC (≈3V) | RTC Battery / +3VLP | RTC Clock, CMOS, RTC Domain |
| +1.0VALW | PWM Controller (Buck Converter) | SoC/PCH Always-On Power Management Domain |
| +1.05VS | PWM Controller (Power Button ke baad) | SoC Main Working Logic |