# Platform Kya Hota Hai?

Platform = CPU Architecture + Chipset (PCH/SoC) + Power Sequence + Signal Design

Chapter-1: Laptop Motherboard Architecture

Intel & AMD Architecture Evolution

| Generation | Year | Architecture | CPU | PCH | SoC | DDR | Examples |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Northbridge + Southbridge | 2004–2009 | 3 Chip | Separate | Southbridge | ❌ | DDR2 | Intel Core2Duo |
| CPU + PCH (Nehalem/Sandy) | 2010–2011 | 2 Chip | Separate | Separate | ❌ | DDR3 | HM55, HM65 |
| CPU + PCH (Ivy Bridge) | 2012 | 2 Chip | Separate | Separate | ❌ | DDR3 | HM76 |
| CPU + PCH (Haswell) | 2013 | 2 Chip | Separate | Separate | ❌ | DDR3L | HM86 |
| Intel BayTrail | 2014 | SoC | Integrated | Integrated | ✅ | DDR3L | Lenovo G40-30 |
| Intel Braswell | 2015 | SoC | Integrated | Integrated | ✅ | DDR3L | HP Stream |
| Intel Skylake | 2015 | CPU + PCH | Separate | Separate | ❌ | DDR4 | HM170 |
| Intel Kaby Lake | 2016 | CPU + PCH | Separate | Separate | ❌ | DDR4 | HM175 |
| Intel Coffee Lake | 2017 | CPU + PCH | Separate | Separate | ❌ | DDR4 | HM370 |
| Intel Comet Lake | 2019 | CPU + PCH | Separate | Separate | ❌ | DDR4 | U-Series |
| Intel Tiger Lake | 2020 | SoC | Integrated | Integrated | ✅ | DDR4/LPDDR4 | Modern Ultrabooks |
| Intel Alder Lake | 2021 | SoC | Integrated | Integrated | ✅ | DDR5/LPDDR5 | 12th Gen |
| Intel Raptor Lake | 2022 | SoC | Integrated | Integrated | ✅ | DDR5 | 13th Gen |
| AMD Ryzen | 2017–Present | SoC | Integrated | Integrated | ✅ | DDR4/DDR5 | Ryzen 3/5/7/9 |

| Architecture | Difficulty | Repair Level |
| --- | --- | --- |
| Northbridge + Southbridge | ⭐⭐ | Easy |
| CPU + PCH | ⭐⭐⭐ | Medium |
| Intel SoC | ⭐⭐⭐⭐ | Advanced |
| AMD Ryzen SoC | ⭐⭐⭐⭐⭐ | Expert |

# Aapke Learning ke Liye Roadmap

Main suggest karunga ki hum isi order me padhein:

### Phase-1 (Architecture) ✅

- Motherboard Evolution

- Northbridge / Southbridge

- CPU + PCH

- SoC

### Phase-2 (Hardware)

- Charger IC

- DC-DC Converter

- PWM Controller - Pulse Width Modulation Controller

- VRM Controller - Voltage Regulator Module Controller

- LDO

- MOSFET

- Coil

- PMIC

### Phase-3 (Power Sequence)

- G3 → S5

- S5 → S0

- RSMRST#

- SUSCLK

- SLP Signals

- VR_ON

### Phase-4 (Practical Repair)

- No Power

- Auto ON

- No Display

- No Charging

- No Boot

Aapne pichhle 2–3 din me jo questions puche hain, unse mujhe lag raha hai ki ab aap signal yaad nahi karna chahte, balki unka source samajhna chahte hain.

# PMIC = Power Management Integrated Circuit

Simple definition:

PMIC ek aisa IC hai jo ek hi chip ke andar kai PWM Controllers, LDOs aur Power Sequence Logic ko integrate karta hai.

Matlab:

PMIC = Multiple Power Controllers in One IC

# Pehle ka Design

Purane laptops me:

Yahan 5 alag IC lage hue hain.

# PMIC Design

Naye boards me: