Is sequence ko ek sentence me samajhiye

| Step | Message |
| --- | --- |
| ACIN | SIO: "Adapter lag gaya." |
| AC_PRESENT | PCH: "AC Power available hai." |
| EC_RSMRST# | PCH: "Standby complete, ab reset se bahar aao." |
| PM_PWRBTN# | PCH: "User ne Power Button daba diya." |
| PM_SLP_S5#/S4#/S3# | PCH: "Ab power rails ON karo." |
| SYSON / EC_ON / SUSP# | SIO: "Main power sequence execute karo." |

Ek Important Baat

Ye Intel platform ka generic sequence hai aur 90–95% Compal, Quanta, Wistron, Inventec, LCFC, Dell, HP, Lenovo, Acer boards isi logic ko follow karte hain. Signal ke naam alag ho sakte hain, lekin kaam aur sequence lagbhag same rehta hai.