Dual 3V and 5V

Check list

Part 1 Generate LDO

- VCC 19 v

- ENLDO(enable LDO)  By voltage devider

- LDO3/VREG3( send to SIO), LDO5/VREG5 (3VL OUTPUT)/ (5VL OUTPUT)

Part2

- ENM (enable master from SIO) 3volt

- ENTRIP1, ENTRIP2

- BOOT1, BOOT2

- UGATE1, UGATE2

- LGATE1(should be low hight(3,4, 5 hoga to switching start nhi hoga)), LGATE2

- PHASE1(mosfet output voltage feedback), BOOT1, UGATE, LGATE – internal process

- FB1

- PGOOD(output pin sending info this department is working fine) 3volt

Note

3/5V Regulator IC - LDO and Switching  voltage out krta hai…

Difference LDO – Low ampier and  Switching Voltage(VALW) – high empier

LDO (Low Dropout)

StepDown IC se out hota jo kafi jagah jata hai.

Main kaam SIO and PCH ko jagana intialize krna hota hai

Single IC 3V

Shortcut to find by enldo

Check List

Part 1 LDO Generate

- In(VCC)

- EN2 (=ENLDO)– by voltage devider/ ise self bhi generate kr sakati hai

- LDO- 3VLP

("+3VLP" refers to a +3.3V Low Drop-Out (LDO) It is a critical, always-on voltage used to power the Embedded Controller (EC) and initialize the system before main power rails activate)

Part 2

- EN1  ~3volt – (from SIO)

First SIO Requiremet puri karni hai

3VLP distribution  search krenge

(issi se on/off hota hai)

Note:

3 volt regulator suppy se pehle LDO  banta hai

https://www.youtube.com/watch?v=HUelstoHB3k