Types of Diode

Location Letter – D, PD, Q, PQ

Note Jese Zener Diode kaam krta hai wese ye Schottky diode bhi kaam krta hai.

Dono side se votage pass krta hai

Note Forward Bias me as a diode kaam krta hai full voltage pass krta hai.

Reveses Bias me Voltage ko rokta hai..

OK Condition

Meter on diode Mode

Red probe Anode and Black Prob Cathode

Reading  value 200-600 depends on diode

Red probe Cathode and Black Probe Cathode – No Reading

Diode Testing

Diode ek one-way component hai – ek direction me current jaane deta hai, dusri me rok deta hai.

Testing se pata chalta hai diode OK, Open, Short ya Leakage me hai.

- Open → Diode bilkul conduct nahi karta

- Short → Diode dono direction me conduct karta

- Leakage → OFF hone par bhi thoda current pass karta

## Testing Method (Multimeter – Diode Mode)

### ✅ Good Diode

Healthy value 100+

❌ Open Diode

❌ Short Diode

⚠️ Leakage Diode

Quick Table (Easy yaad)

| Condition | Forward | Reverse |
| --- | --- | --- |
| Good | 0.4–0.7V | OL |
| Open | OL | OL |
| Short | 0 / Beep | 0 / Beep |
| Leakage | OK | Value aa rahi |

- OL = Open

- 0 dono side = Short

- Reverse me value = Leakage