### CLR ka matlab

Laptop motherboard me CLR ka matlab hota hai “Clear / Cleaning related component”
Ye mainly signal line ya voltage line ko clean, filter aur reset condition me lane ke kaam aata hai.

CLR component = capacitor / resistor / jumper jo kisi line ko clean (noise remove) ya reset state me laata hai.

### Fusible Resistor kya hota hai?

Fusible Resistor ek aisa resistor hota hai jo resistor + fuse dono ka kaam karta hai.
Normal time me current control karta hai, aur fault aate hi jal kar open ho jata hai – circuit ko bacha leta hai.

Testing Multimeter se:

| Mode | Reading |
| --- | --- |
| Good | 0Ω – 5Ω |
| Blown | OL / No beep |

OL ka matlab 👉 Open Line / Over Limit

🔹 Multimeter me OL ka meaning

OL = Circuit open hai, current flow nahi ho raha

Matlab:

- Connection tootā hua hai

- Ya resistance bahut zyada hai (limit se bahar)

🔹 Diode testing me OL

- Reverse direction me OL → Diode OK

- Dono direction me OL → Diode OPEN (dead)

🔹 Resistance mode me OL

- OL → Infinite resistance

- Continuity nahi

🔹 Real world example

Jaise wire kat gayi ho –
current flow possible nahi → OL

Troubleshooting
Sabse pehle

Impedence/GR value

Check krte hai