Lenovo G40-30 / NM-A311 Rev 1.0 ki schematic

Is board ki kuch important information:

- CPU: Intel BayTrail-M (BGA1170)

- EC/SIO: ITE IT8586E

- BIOS: 8MB SPI Flash

- DDR3L Memory

- BayTrail SoC (PCH CPU ke andar hi integrated hai, alag PCH IC nahi hai.) Lenovo G40 30 NM-A311 REV 1.0 Schematics.pdf

### Is schematic se hum kya kya seekhenge

Main is board se complete repair course jaisa sequence nikal sakta hoon:

- ✅ Charger Section

- ✅ 3V/5V ALW Generation

- ✅ RTC Circuit

- ✅ SIO Inputs

- ✅ SIO Outputs

- ✅ BIOS Communication

- ✅ Power Button Sequence

- ✅ SUSCLK

- ✅ PM_RSMRST#

- ✅ SLP_S5#, SLP_S3#

- ✅ CPU Power Rails

- ✅ Display Sequence

# Repair Engineer Rule

Agar aap dead motherboard troubleshoot kar rahe hain, to Power Button dabane se pehle har board par lagbhag ye hi check karte hain:

- ✅ 19V

- ✅ Charger IC

- ✅ +3VALW / +5VALW

- ✅ RTC_VCC

- ✅ 32.768 kHz

- ✅ EC/SIO VCC

- ✅ EC_RST#

- ✅ BIOS Communication

- ✅ ACIN#

- ✅ LID_SW#

- ✅ Thermal / Protection Signals

- ✅ SIO Ready

Iske baad hi Power Button ka signal process hota hai.

SIO Basic Requirement (Before Power Button)

# SIO ki Minimum Basic Inputs

| Signal | Required | Function |
| --- | --- | --- |
| ✅ VCC / VSTBY0 (+3VALW) | Yes | SIO Power |
| ✅ RTC_VCC / VBAT | Yes | RTC Power |
| ✅ 32.768 kHz Crystal | Yes (board dependent but recommended) | RTC Clock |
| ✅ EC_RST# | Yes | SIO Start |
| ✅ SPI BIOS | Yes | Firmware Execute |
| ✅ ACIN# | Yes | Adapter Detect |
| ✅ LID_SW# | Optional for boot, but important | Lid Detect |
| ✅ Thermal Inputs | Protection | Temperature Monitoring |

Agar SIO ki ye 8 basic requirements complete nahi hongi, to:

  ❌ Power Button ka response nahi milega.

  ❌ PBTN_OUT# generate nahi hoga.

  ❌ Aage ka power sequence (SUSCLK, PM_SLP_S5#, PM_SLP_S3#, etc.) start nahi hoga.

Power Button Press ke Baad Complete Sequence

User Presses Power Button

│

▼

1. ON/OFF Signal → SIO

│

▼

2. SIO Debounce (10~50ms)

│

▼

3. SIO checks:

├─ ACIN#

├─ RTC OK

├─ BIOS Running

├─ Thermal OK

├─ Lid Status

└─ No Protection

│

▼

4. PBTN_OUT#

│

▼

5. SoC detects Power Button

│

▼

6. SUSCLK Start

│

▼

7. PM_RSMRST# Release

│

▼

8. PM_SLP_S5# HIGH

│

▼

9. SYSON / EC_ON

│

▼

10. +3VS / +5VS

│

▼

11. PM_SLP_S3# HIGH

│

▼

12. Main Power Rails ON

(+1.8V, +1.05V,

VCCSA, VCCIO,

CPU Core ...)

│

▼

13. PLT_RST#

│

▼

14. BIOS Read by SoC

│

▼

15. POST

│

▼

16. Display

## Repair Rule

| Signal | Kaun Generate Karta Hai? | Kahan Measure Karte Hain? |
| --- | --- | --- |
| PBTN_OUT# | SIO (EC) | SIO output pin |
| SUSCLK | SoC (internal PM logic) | SoC output pin |
| PM_RSMRST# | SoC | SoC output pin |
| PM_SLP_S5# | SoC | SoC output pin |
| PM_SLP_S3# | SoC | SoC output pin |
| PLT_RST# | SoC | SoC output pin |