Concept

EC (SIO) aur PCH do alag IC hain.

- EC (SIO) = House Manager (Power Button, Keyboard, Fan, Battery...)

- PCH = Main Controller (CPU, RAM, PCIe, USB...)

Laptop ON karne ke liye dono ka aapas me "baat karna" zaruri hai.

Step 1 : Adapter Connect

+3VALW

+3VRTC

+RTCVCC

EC aur PCH dono ko standby power milti hai.

Lekin dono abhi fully ON nahi hue.

Step 2 : PCH RTC Start

RTC Crystal

│

▼

PCH RTC

│

▼

SUSCLK

│

▼

EC

SUSCLK kya karta hai?

Ye PCH bolta hai

"Main zinda hoon."

EC ko timing reference mil jata hai.

Step 3 : EC Ready

EC ke paas

- Supply

- Reset

- Clock

- BIOS

sab aa gaya.

Ab EC firmware execute karta hai.

Step 4 : Power Button

Power Button

│

▼

EC Detect

User ne Power Button dabaya.

EC samajh gaya.

Step 5 : EC PCH ko Request bhejta hai

EC bolta hai

Please Laptop ON karo

PCH request receive karta hai.

Step 6 : PCH Ready hai ya nahi?

Agar PCH Ready hai

to

RSMRST#

Release karega.

Ye signal bolta hai

"Main Reset se bahar aa gaya."

Step 7 : Sleep Signals

Fir PCH ek-ek karke release karta hai

SLP_S5#

│

▼

SLP_S4#

│

▼

SLP_S3#

Har signal ek naya regulator ON karta hai.

Step 8 : PCH Reset

PCH khud bhi reset se bahar aata hai.

Aur dusre devices ko bhi reset release karta hai.

Poora Communication

Ab samajhiye har signal ka matlab

1. SUSCLK

PCH → EC

Meaning

Main Ready hoon.

2. RSMRST#

PCH → EC

Meaning

Main Standby Reset se bahar aa gaya.

3. SLP_S5#

PCH → Motherboard

Meaning

Soft OFF khatam.

Power Sequence Start.

4. SLP_S4#

Meaning

Hibernate se bahar.

Aur regulators ON.

5. SLP_S3#

Meaning

Sleep khatam.

Ab System ON ki taraf jao.

Repairing Point

Isliye hum ye signals check karte hain:

✅ SUSCLK hai?

↓

✅ RSMRST# High hai?

↓

✅ SLP_S5# Release hua?

↓

✅ SLP_S4# Release hua?

↓

✅ SLP_S3# Release hua?

Agar chain kisi ek signal par ruk gayi, to usi point ke aage ki power sequence bhi ruk jayegi.

EC–PCH Handshake & Power Sequence Confirmation"

"EC ↔ PCH Communication Confirmation"

  SUSCLK, RSMRST#, SLP_S5#, SLP_S4#, SLP_S3# zyadatar PCH → EC/Board ke outputs hote hain.

  EC se PCH ki taraf PWRBTN#, SMI#, SCI#, KBC signals jaate hain.

Note:

Purane Intel Platforms (ICH / HM Series)

Kai purane Intel boards me:

EC

│

RSMRST#

│

▼

PCH

Yahan EC RSMRST# release karta hai, jisse PCH standby reset se bahar aata hai.

Isliye aapne jo padha hai, wo galat nahi hai.

Naye Intel Platforms (8th Gen+, Tiger Lake, Alder Lake...)

Aksar sequence aisa hota hai:

RTC Power

│

▼

PCH

│

▼

RSMRST#

│

▼

EC aur baaki circuits

Yahan PCH RSMRST# control karta hai.

RSMRST# ki direction assume mat karo.

Hamesha schematic dekho.

- Agar net EC pin se output hai → EC → PCH

- Agar net PCH pin se output hai → PCH → EC/Board

Aapke Notes ke liye

Main is point ko aise likhunga:

RSMRST#

Purpose:

Standby Reset Signal.

Ye signal PCH ko ya PCH se generate ho sakta hai

(board/platform dependent).

Iska kaam Standby Mode se Power Sequence ko start karna hai.

Direction hamesha schematic se confirm karein.