1️- Series Resistor

Resistor load ke line me lagta hai.

👉 KYU lagate hain?

- Current limit karna ho

- Voltage drop chahiye ho

- Load ko protect karna ho

👉 Kya effect hota hai?

- Current kam hota hai

- Load par voltage kam milti hai

👉 Real Life

- LED ke saath resistor

- IC input protection

👉 Series = rokna / limit karna 🚦

2️- Parallel Resistor

Resistor side me lagta hai (extra path).

✅ Reason 1: Current share karne ke liye

Ek resistor par zyada load ❌
2 parallel resistor → current divide ✔

👉 Power handling badh jaati hai 🔥

--------------------------------------------------------------------------------------------------------------------------------------

1️ Series Resistor

series me lagayenge to jyada voltage drop hogi but current same rehega

✅ Bilkul Correct 👍

🔹 Explanation

- Series circuit me sirf ek hi current path hota hai

- Isliye har component me SAME current flow karta hai

- Har resistor apne value ke hisaab se voltage drop karta hai

- Current = 12 / (4+2) = 2A

- Voltage drop =

  - 4Ω → 2×4 = 8V

  - 2Ω → 2×2 = 4V

✔ Current same, voltage divide ✔

2️- Parallel Resistor

👉 Aapka statement:

parrallel me voltage same rhegi but current devide ho jayega

✅ Ye bhi Bilkul Correct 👍

🔹 Explanation

- Parallel circuit me same voltage source sab branch ko milta hai

- Isliye har resistor par SAME voltage hoti hai

- Lekin resistance alag hone ki wajah se current divide ho jata hai

- 6Ω branch → 12/6 = 2A

- 3Ω branch → 12/3 = 4A

✔ Voltage same, current divide ✔

Note:

👉 Total current = 2A + 4A = 6A

⚠️ Yahan voltage add nahi hoti,
sirf current add hota hai.

🔑 Big Clarification (MOST IMPORTANT)

- ❌ “−2 voltage” jaisa kuch nahi hota yahan

- ✔ GND par voltage 0V hi rehti hai

- ✔ Jo add hota hai wo current hota hai, voltage nahi

Parallel me:
– Same nodes ⇒ Same voltage
– Alag resistance ⇒ Alag current
– Node par milte waqt ⇒ Current add hota hai

Golden Rule (Exam / Practical ke liye)

- Series:
👉 Current = same
👉 Voltage = divide

- Parallel:
👉 Voltage = same
👉 Current = divide

One-Line

Series = Voltage batta, Current same
Parallel = Current batta, Voltage same