VIN section motherboard ka wo hissa hota hai jahan adapter se aane wali main voltage (≈19V) board me enter hoti hai aur safety check ke baad aage pass hoti hai.

## ❓ VIN Section kyu hota hai? (Reason)

VIN section isliye hota hai taaki:

1️⃣   Reverse polarity se protection
– Galat charger lag gaya to board safe rahe

2️⃣   Short / surge protection
– Agar aage short ho, to 19V direct board me na jaye

3️⃣   Controlled power entry
– Power tabhi jaaye jab conditions OK ho

4️⃣   Battery–Adapter isolation
– Adapter aur battery ek dusre ko damage na karein

VIN Sectionko Charging IC Control karti hai.

Note
No charching concept

There are two types of  VIN Section – N Channel, P Channel

Latest pattern

1st Mosfet

P – Channel

In – Source – 19.5

Out – Drain – 19.5

Gate – 6.4  	      P – Channel

Note Gate par input se kam volt G < Input/Source

2nd Mosfet

In Drain – 19.5

Out Source – 19.5

Gate – 1.7 	      P – Channel

Ab supply 19.5 volt CLR par jati hai

Note

Type C charging hai to board me ke andar  9/12 volt supply hi milegi 19 / 5 volt nhi milegi

Note

N channel ke Gate par input /Drain se jyda volt hote hai…

Gate  > Input/ Source

1st  Mosfet

Input Drain  -19.5 volt

Output Source – 19.5 Volt

Gate – 25volt

2nd Mosfet

Input Source – 19.5 Volt

Output Drain – 19.5 volt

Gate – 25Volt

Then the voltage goes to CLR

Agar Charging Jack hai to Vin Section melga hi

And board me 19 volt milega q ke type C charging nhi hai.

Kisi kisi board me clr agar battery section ke pass hai to 19.5 volt nhi dega board ko vo 12/14volt dega.

Query

1️⃣    Concept – Mosfet OFF hone ke baad bhi voltage kaise mil sakta hai?

MOSFET OFF hone ka matlab “current flow band” hota hai,
voltage presence band hona zaroori nahi.

Voltage leak / pre-charge / diode path se aa sakta hai.

Outpt par voltage hona ≠ MOSFET ON
ON/OFF decide hota hai current se, sirf voltage se nahi.

## N-Channel MOSFET ka rule

👉 Gate > Source ⇒ MOSFET ON

👉 Gate ≈ Source ⇒ MOSFET OFF

## 1️⃣   Concept – P-Channel MOSFET ka basic rule

P-Channel MOSFET tab ON hota hai jab Gate, Source se LOW ho.
Yani Gate–Source ke beech negative voltage chahiye.

P-Channel MOSFET = LOW-side Gate se ON hota hai
(Source high rehta hai)

🔹 P-Channel

🔹 P-Channel ON condition

🔹 P-Channel OFF condition

MOSFET ko ON ya OFF karne ka asli control “Gate aur Source ke beech ka voltage (Vgs)” karta hai, na ki sirf gate ka voltage.

👉 VGS = VG – VS

| Type | ON condition |
| --- | --- |
| N-Channel | Vgs positive & Vgs ≥ Vth |
| P-Channel | Vgs negative & ¦Vgs¦ ≥ Vth |

About schematics and how read them

https://www.youtube.com/watch?v=RFta96O1Fqg

https://www.youtube.com/watch?v=gzTNJr7bRfc