## 🔁 Motherboard Study Roadmap (VIN ke baad)

### 1️⃣   +3VALW / +5VALW Section (ALWAYS ON Power) ⭐⭐⭐

Concept:
Ye rails adapter lagte hi turant aati hain.

Kya practice karein:

- Buck IC (PU) identify karna

- Coil (PL), MOSFET (PQ), feedback resistors

- EN pin, VIN pin, VOUT pin trace

Why next?
👉 Agar 3V/5V nahi hai = aage ka kuch bhi ON nahi hoga

### 2️⃣   EC (Embedded Controller) Power Section ⭐⭐⭐

Concept:
EC motherboard ka brain hota hai.

Study points:

- EC_VCC / +3VALW se power

- EC_RST#

- EC_CLK

- Power button signal (PWRBTN#)

Practice:

- Power button press karne par kaunsa signal change hota hai

### 3️  SUS / SLP Signals Section ⭐⭐

Concept:
Sleep se wake-up ka process.

Important signals:

SLP_S5#

SLP_S4#

SLP_S3#

Logic:

- Ye HIGH hote hi next voltages ON hoti hain

### 4️⃣   Secondary Rails (CPU ke liye) ⭐⭐

Concept:
CPU/GPU/RAM ke liye alag-alag power.

Examples:

- +Vcore

- +VCCSA

- +VCCRAM

Practice:

- Enable signal kis se aa raha

- VRM controller ka role

### 5️⃣   BIOS Section ⭐⭐

Concept:
Firmware + EC communication.

Check points:

- BIOS VCC (1.8V / 3.3V)

- SPI_CLK, SPI_CS#, SPI_DO/DI

- BIOS corrupt vs no-power difference