Voltage Dividers

Voltage Divider ek resistor base circuit hota hai, jo bade voltage ko chote voltage me change kr deta hai.

Sirf 2 resister ko series me jodkar ek output voltage bna sakte hai.

Note: (voltage devider is used as a reference)

Output voltage 2.5 volt par load nhi de sakate keyu ke iski ampier rating bahut kam hoti hai. Iska use sirf  reference ke liye use kr sakate hai

What is the difference between Reference and Main Voltage

Reference(hight resister )- kisi kaam ko krwana

Main(low resister) – kisi kaam ko khud krna

Requirement – register in series, vcc/vdd, Gnd

Vcc – voltage colector to collector- voltage collector se aa rha hai ya collection par/ko ja rha hai

VDD- voltage Drain to Drain – voltage Drain se aa rha hai ya Drain par/ko ja rha hai.

To calculate

https://ohmslawcalculator.com/voltage-divider-calculator

Pull Up

Ka use data/ singnal ko ek ic se dusri ic me bejne ki liye use hota hai..

Pull Down

Instatly off ho jaye

Pull-up Resistors

👉 Concept

Resistor signal ko VCC ki taraf kheechta hai

👉 Case

- Button open → Signal = VCC (5V)

- Button press → Signal = 0V

Signal ko high level par rakhane ke liye (us line ko contineously vohi voltage mile)

Pull-Down Resistor – Voltage

👉 Concept

Resistor signal ko GND ki taraf kheechta hai

👉 Case

- Button open → Signal = 0V

- Button press → Signal = 5V