SIO Output Post Power Button Pressed

Note: Power Button Signal First SIO ke Pass Jata hai…(sio ko pata chal jata hai button pressed hua hai).

- Button Press ke bad First Singnal SIO EC_ON# / S5_Enable#(if applicable) singnal 3/5 Buck Converter ko send krega to generte 3 & 5Volt supply.

- SIO Se Second Signal RSMRST# (PCH Reset Signal Jese EC_ ka – ECRST#) Release hota hai,  PCH ko.

RSMRST# ke bad PCH Active ho jayega aur First Signal Release karege SUS_CLK.

- Third Signal PM_PWRBTN# (Power Button Out) SIO Se Release hota hai PCH ko.

Note: jab SIO ON/OFF low hai to PM_PWRBTN# bhi low, jab ON/OFF par high to PM_PWRBTN# bhi high. SIO, PCH dono active ho gye

- Jese hi PCH PM_PWRBTN# ko high milega to PCH

PM_SLP_S4#

PM_SLP_S3#

Release kr ke SIO ko dega