AC_Present

| Signal | Direction |
| --- | --- |
| ACOK / AC_PRESENT | Charger IC → SIO |
| AC_PRESENT / ACIN# | SIO → PCH |

| Signal | Kis ko jata hai | Kaam |
| --- | --- | --- |
| ACIN | SIO (EC) | SIO ko batata hai ki Adapter connect hai. |
| AC_PRESENT | PCH (aksar SIO ke through ya direct) | PCH ko batata hai ki AC Power available hai. |

Har motherboard me routing same nahi hoti:

- Kuch boards me Charging IC → SIO → PCH hota hai.

- Kuch boards me Charging IC → PCH direct bhi hota hai.

- Kuch boards me signal ka naam ACOK, AC_PRESENT, ACIN, ACDET ya ADP_PRESENT bhi ho sakta hai.

ACIN and AC_PRESENT

1. ACIN

ACIN sirf tab active hota hai jab Adapter connected ho.

Adapter ✔️  → ACIN = HIGH
Battery ❌  → ACIN = LOW

Reason:
ACIN ka matlab hi hai "AC Adapter Input Detect".

2. AC_PRESENT

Ye board design par depend karta hai.

Case 1 (Most Common)

Adapter ✔️ → AC_PRESENT = HIGH

Battery Only ❌ → AC_PRESENT = LOW

PCH ko pata chal jata hai ki system adapter par chal raha hai.

Case 2 (Kuch Modern Boards)

Battery par bhi PCH boot kar sakta hai.

Us case me PCH ko battery ki information alag signal se milti hai.

Jaise:

- BAT_PRESENT

- BATLOW#

- SMBus (SCL/SDA)

- Battery Fuel Gauge IC

Isliye AC_PRESENT LOW hone ke baad bhi laptop battery se ON ho jata hai.

Ek Important Point

Laptop battery par bhi normally ON ho jata hai, kyunki PCH ko sirf AC_PRESENT par depend nahi rehna padta. Battery ki information Battery Charging IC aur SMBus (SCL/SDA) ke through bhi milti hai.

Repair ke liye yaad rakhiye:

- ACIN = "Adapter detect."

- AC_PRESENT = "System AC mode me hai."

- Battery mode me AC_PRESENT zaroori nahi HIGH ho. PCH battery ki information dusre signals se bhi le sakta hai.