👉 Laptop repairing me “impedance” koi alag cheez nahi hoti
👉 Ye bas current ke raste me rukawat ka naam hai

## 1️Line:

“Impedance = current ke flow ka total opposition”

### 👉 Comment (simple words):

Current chalna chahta hai,
circuit usko jitna roke = impedance

Jaise:

- Raasta seedha → kam rukawat

- Raasta kharab → zyada rukawat

## 2️Line:

“Sirf resistance hi nahi, AC signals me capacitor + inductor ka effect bhi add hota hai”

### 👉 Comment:

Circuit me 3 cheeze current ko rokti hain:

| Part | Kya karta hai |
| --- | --- |
| Resistor | Hamesha roke |
| Capacitor | Fast change ko roke |
| Inductor | Current change hone se roke |

Inductor

“Current change”

👉 Matlab:

- Abhi current 0 hai

- thodi der baad 5 ho gaya

- ya 5 se 0 ho gaya

Is 0 → 5 ya 5 → 0 ko hi kehte hain
👉 current change

Inductor ko kya problem hoti hai?

Inductor ko current ka achanak badalna pasand nahi.

- Current achanak badhe → inductor rokta hai

- Current achanak ghatne → inductor rokta hai

👉 Isliye bolte hain:

Inductor current change ko oppose karta hai

🔹 Case-1: Switch ON

- Switch ON kiya

- Current turant 0 → max jana chahta hai

- Inductor bolta hai: “ruk, dheere-dheere”

👉 Current slowly increase hota hai

🔹 Case-2: Switch OFF

- Current turant band hona chahta hai

- Inductor bolta hai: “ruk, main chalne dunga”

👉 Inductor reverse voltage bana deta hai
👉 Current dheere-dheere zero hota hai

Laptop motherboard se connect karo 🔧

🔹 Buck converter me

- MOSFET ON–OFF hota rehta hai

- Current bar-bar change hota hai

👉 Inductor smooth karta hai
👉 CPU / RAM ko stable current milta hai

👉 Matlab:

- Current achanak badhe → inductor slow kare

- Current achanak ghate → inductor current chalata rahe

👉 Isliye current smooth flow me rehta hai.

Resistor – “Hamesha roke”

👉 Kya karta hai?

Resistor current ko hamesha limit karta hai
chahe DC ho ya AC.

👉 Matlab:

- Current zyada aa raha → resistor kam kar dega

- Current kam aa raha → resistor fir bhi limit karega

👉 Resistor ko farq nahi padta current fast hai ya slow

Capacitor – “Fast voltage change ko roke”

Capacitor voltage ko stable rakhta hai.
Voltage achanak badalne lage to rokta / smooth karta hai.

👉 Matlab:

- Voltage upar jaaye → capacitor charge le leta

- Voltage niche jaaye → capacitor charge de deta

👉 Isliye:

Capacitor voltage ka shock absorber hai

Capacitor voltage ka achanak badalna pasand nahi karta.

👉 Matlab:

- Voltage achanak upar jaaye → capacitor absorb karega

- Voltage achanak niche aaye → capacitor supply karega

👉 Isliye capacitor voltage smooth karta hai.

One-line FINAL summary 🔥

Resistor = current ka speed breaker
Capacitor = voltage ka shock absorber
Inductor = current ka shock absorber

Resistance,  capacitor , inductor

👉 Jab ye teenon milkar roken,
to uska naam IMPEDANCE.

One-line FINAL understanding (yaad rakhna)

🔑 Resistance + capacitor + inductor milkar
current ko jitna roken = impedance

Laptop repair me:

🔧 Impedance ka matlab =
short hai ya nahi, bas itna