NTC Thermistor ek temperature-sensitive resistor hota hai jo starting surge current ko control karta hai.
Cold state me resistance jyada hota hai, garam hote hi resistance kam ho jata hai.

NTC = Negative Temperature Coefficient
Matlab temperature badhne par resistance kam hota jata hai.

Use Case + Connection Syntax

SMPS / Inverter ke AC input line me series me

Surge Current

Note:- Surge ka matlab hota hai – achanak bahut jyada current ka flow hona, jo normal se kai guna zyada hota hai, sirf power ON karte hi.

Short Definition

Surge Current = Starting time par aane wala extra high current spike.

Use Case + Example

Jab hum SMPS / Charger ON karte hain:

Real World Relation

| Without NTC | With NTC |
| --- | --- |
| Fuse ud jata hai | Fuse safe |
| Bridge rectifier burn | Circuit safe |
| Capacitor damage | Long life |

👉 One Line yaad rakho:
Surge = ON hote hi aane wala dangerous high current jhatka.