SYS_ON se banne wali Supply

RAM Main Power (VDDQ) –  Schematic Search Keyword VDDQ

Ram Regulator Supply VDDQ 1.3V

SYS_ON

↓

RAM Main Power (VDDQ)

DDR3 = 1.5V

DDR3L = 1.35V

DDR4 = 1.2V

DDR5 = 1.1V

↓

RAM Chips

↓

CPU/SoC Memory Controller

✅ VDDQ RAM Ics/Chip ko power deti hai, aur CPU/SoC ka Memory Controller us RAM se communicate karta hai.

SUSP se banne wali Supply (Suspend Domain)

- DDR VTT = VDDQ / 2
(RAM Termination Voltage)

- VREF- generated by Ram ic
(RAM Reference Voltage)

- +1.05VS / VCCSA / VCCPRIM / PCH/SOC Secondary Rail
(Platform Dependent)

Ye sirf SoC ke kuch internal blocks ko power deti hai.

- Power Management Logic

- Memory Controller

- I/O Logic

Kyu?

Main SoC Working actually tab hoti hai jab:

VR_ON
	↓
CPU VCORE
        ↓
CPU Execute BIOS

Ye har board me same nahi hota.

Better likhiye:

+1.05VS
↓
SoC Secondary Power Domain
or
PCH Secondary Power Domain
(Platform Dependent)

Main SoC Working

(Power Button ke baad)

Final Notes (Professional)

SYS_ON (S0 Domain),  S3 (SUSP) Power Domain

| Supply | Purpose |
| --- | --- |
| DDR VDDQ | Main RAM Power |
| LCD Power | Display Panel |
| Audio Power | Audio Codec |
| Camera Power | Webcam |
| WLAN Power | Wi-Fi |
| USB Power | USB Devices |
| Touchpad Power | Touchpad |
| Keyboard Backlight | KB Backlight (if available) |

SUSP (Suspend Domain)

| Supply | Purpose |
| --- | --- |
| DDR VTT | RAM Termination |
| DDR VREF | RAM Reference |
| +1.05VS / VCCSA / VCCPRIM | SoC/PCH Secondary Logic |
| Other Suspend Rails | Platform Dependent |