- Adapter ID Pin

Charger ID PIN  SIO ko batati hai ki kitne WATT  45/65 ki adapter lagi hui hai.

- Agar CMSRC nhi detect hui to, ACDRV banna band ho jayegi.

- Battery charging ke liye LDO banana Jaruri hai.

- Battery Charging Requirements

BIOS (LDO 3.3 v honi chahiye pull ke through)

SIO (LDO3.3 v honi chahiye pull ke through)

Battery Present (LDO honi chahiye pull ke through - high hai to low aur low hai to high)

- SM_DATA (LDO3.3 v honi chahiye pull ke through)

- SM_SCL(LDO3.3 v honi chahiye pull ke through)

- BI (LDO3.3 v honi chahiye pull ke through).

- AC OK (LDO3.3 v honi chahiye pull ke through)

Note SIO ACOK ko read krta hai agar 3.3 volt hai to adapter connected aur 0volt to adapter not connected, battery se chalega.

✅ Sahi logic:

Adapter 19V

↓

Charger IC

↓

LDO 3.3V

↓

SIO / BIOS Active

↓

ACOK

↓

SMBUS Communication

↓

Battery Detect

↓

Charging Enable

↓

Battery Charging

Aur ye points bhi important hain:

- ACOK = 3.3V → Adapter Connected

- ACOK = 0V → Adapter Not Connected

- SM_DATA aur SM_SCL par 3.3V pull-up hona chahiye

- Battery Present signal valid hona chahiye

- BIOS aur SIO powered hone chahiye