Types – N chanel, P Channel, Diode

- VCC 19V

- ACDET ~2.4-3.0V (as per voltage devider) or as a enable

- REGN 6V  (ic start internal work ok)

- ACDRV (to switch on mosfet)

- CMSRC – mosfet on feedback

- AC OK `3.3 volt pull ke through – sio feedback( send msg to sio ki power aa rha hai)- ACDET is must

How to Find VIN Section in Schematics

ACP, ACN – Current measurement of CLR

Regn

VCC/DCIN – IC Power Supply

ACIN, ACOK – Output Pins (3.3 volt pull-through)

(Send msg to SIO that adapter supply CLR par pahuch gya hai(adapter or battery supply is stable)

ACDET – 2.6-3.0V Valid adapter insert hai ya nhi…

Note:  ACDET nhi hoga to ACOK release nhi hoga, ACOK ka apna volt LH nhi hota hai, Pull ke through hota hai..

ACDRV – mosfet ko on krne keliye.

CMSRC – dono se source se connected, mosfet switch on feedback

REGN ~6V  -VCC/ACDET valid hai to, (IC internally ok hai to)

SRP, SRN – CLR for battery current measurement ke liye.

NOTE- P Channel Concept

P-Channel MOSFET me ACDRV nhi hota hai kyu ke 25 not required

P-Channel MOSFET me CMSRC nhi hota hai

P channel low  voltage = ON

Note: DC Jack voltage CLR Voltage ke equal hona chahiye…

G=S OFF

Random

Open Drain

Close Drain

ACOK – High Active 3.3V

ACOK# - low Active 0v