SIO Requirement Before Power Button Pressed

- ACIN – Charging IC Se Volt in hone ke bad

- +3VLDO-  on sio multiple pins

LID_SW#,  ACIN#(ACOK, ACON)- to confirm – adapter or battery

VSTY0, VSTBY1, VSTBY2, VSTBY3, VSTBY4, VSTBY5, VSTBY6, - input pins

- PWRBTN

EC_RST

SIO POWER ON REQUIRMEN TESTING WITH #SCHKMATIC #DIGRAM TESTING

https://www.youtube.com/watch?v=tHzMVuMxTP8&list=PLlz0cASECY1FZ36h9oYhTPw6jix9cV39a

SIO Input  (~3.3LDO)- basic requirement

#### 1. VCC (EC_VDD / VCC, VSTBY0 /), AVCC +3VALW_EC)

- Status: ✔️ Correct

- Ye SIO ki standby power supply hoti hai.

- Normally 3.3V LDO / 3VALW se aati hai.

- Iske bina SIO bilkul dead rahega.

#### 2. EC_RST – 3VALW_EC by Pull-up

EC_RST# → PCH/Reset Circuit se generate hota hai. Jab 3VALW stable ho jata hai aur reset release hota hai tab EC execute/start hota hai.

#### 3. ACIN#

- Status: Correct

Purpose:

- Adapter detects signal.

- SIO ko batata hai charger connected hai ya nahi.

#### 4. LID_SW#

- Laptop lid open hai ya close hai uski information deta hai.

- Closed hone par SIO sleep initiate kar sakta hai (setting ke according).

Example:

| Lid | Voltage |
| --- | --- |
| Open | ~3.3V |
| Closed | 0V |

LID_SW#  info about latop in sleep mode(0V) or working mode(3.3V)

#### 5. VCIN0_PH1 – EC Thermal (~2.7V)

Ye board specific hai.

Agar schematic me ye VCIN0_PH1 naam se hai to generally:

- EC ka Analogue Input hota hai.

- Voltage divider ke through CPU/GPU thermal ya power monitoring hoti hai.

- Iski value board ke hisab se 2V–3V ho sakti hai.

VCIN0_PH1 → EC Analog Input (Voltage Divider) for Thermal/Power Monitoring (~2.7V board dependent).

SIO Output

- EC_ON / S5_Enable/S5_ON – 3/5 Enable

Note: the above requirement should be fulfilled for EC_ON

ON/Off – 3V - Power Button (H L H)

After the Power Button Pressed EC Release 3 Signal for PCH

PCH INPUT (System Power Management Section )For DDR3

- EC_RSMRST# (~3.2V) For PCH (PCH ko jagata hai)

- PWRBTN_OUT# (~3.2V)- For PCH

- AC_PRESENT (~3.3V)– For PCH System Management

AC_PRESENT – Kuch board me power button press se pehle aur kuch me power button press ke bad out hota hai SIO se.

After receiving the above 3 requirements, PCH Release 3 signals

PCH OUTPUT

- SUSCLK- send to SIO

- PM_SLP_S4# ~3.3V send back to SIO- Release SYSON#

- PM_SLP_S3# ~3.3V send back to SIO – Release SUSP#

SYSON# - AS EN Ram Regulator IC and release VDDQ as per DDR3(1.8/1.35V)/DDR4(1.2V).

SUSP#

- AS EN Ram Regulator IC and Release VTT = VDDQ/2

- Ram Regulator – 1.5 VSP

- 1.8 VGS

- 1.05VS

RANDOM

SIO (EC) Input Signals

1. VCC (EC_VDD / VSTBY0 / +3VALW_EC)

→ Standby 3.3V supply to wake up SIO.

2. EC_RST#

→ Reset signal released after 3VALW becomes stable.

→ Starts EC firmware execution.

3. ACIN#

→ Detects AC adapter connection.

4. LID_SW#

→ Detects whether the laptop lid is open or closed.

5. VCIN0_PH1

→ Analog voltage input (through voltage divider) used for thermal/power monitoring (~2.7V, board dependent).

SYSON, & SUSP - CPU Core se pehle banane wali sabhi supply ko control krte hai

RAM 1.8 V

PCH Supply 1.05V

1.8V

2.5V

3/5 DC to DC

Wistron me syson susp nhi hota hai

Direct SLPS4, SLPS3 PCH se release hokar supply on krte hai.