## Main Sections (Repair ke hisab se)

| No | Section Name | Kaam |
| --- | --- | --- |
| 1 | DC-IN / Charging Section | Adapter se 19V lena, battery charge karna |
| 2 | Always Power Section | +3VALW, +5VALW banana |
| 3 | EC / BIOS Section | Power button, sequencing, system start control |
| 4 | Secondary Power | +1.8V, +1.05V, RAM rails |
| 5 | CPU / GPU Core Section | Vcore, VGFX (high current buck) |
| 6 | Clock & Reset Section | Crystal, clock generator, reset signals |
| 7 | Display Section | LCD power, backlight, LVDS / eDP |
| 8 | I/O Section | USB, Keyboard, Touchpad, LAN |
| 9 | Storage Section | HDD / SSD power & data |
| 10 | Audio Section | Speaker, Mic, codec IC |

Agar laptop bilkul dead hai →
Repair hamesha DC-IN → Always Power → EC sequence se start hoti hai.
Is order ko follow karega to fault fast milega. 🔧

## Identify karne ke Rules (Use-Case)

| Kya dikhe | Matlab kaunsa section |
| --- | --- |
| Coil + 2 MOSFET + PWM IC | CPU / GPU Buck Section |
| Coil + 1 MOSFET + diode | Normal Buck (3V / 5V) |
| 3-pin IC likha ho 1.8V / 2.5V | LDO Section |
| “VIN, ACDET, CMSRC” pins | Charging Section |
| Voltage name: +3VALW, +5VALW | Always Power Section |
| “EC”, “KBC”, “KB9012” IC | EC Section |
| BIOS chip 8-pin | BIOS Section |
| “LCDVDD, BL_EN” | Display Section |