- No Power LA-C701P No Power  (issue in RTC) – Troubleshoot with Schematics

https://www.youtube.com/watch?v=Lrojqjk31GU&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=19

Step-1 RTCVCC first as per schematics

Schematics – Follow Power Sequence(almost available in all schematics)

RTCVCC to PCH_RTCRST# Line capacitor leakage

- No Power (Issue in VIN Section) - troubleshoot without Schematics

https://www.youtube.com/watch?v=GxNDFR3zo0Q&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=18

- Vin Sec - not ok (first mosfet gate no volt)

- 6050A2723701-MB-A02 No Power  (issue in Lid Switch) -  troubleshoot without Schematics

https://www.youtube.com/watch?v=eSTA-TNYd30&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=17&t=11s

- Vin -ok

- 3/5 volt -ok

- Check Either RTC or Lid Switch (3.3V in – 3.3V out if lid open)

Magnet field se voltage out nhi hoga

Lead Switch-not ok

4- NO Power NM-C561 REV.1 Troubleshoot with Schematics   https://www.youtube.com/watch?v=xa_Jr1sest0&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=16&t=79s

Step-1 – Check VIN -OK

Step-2 – Check 3/5V ALW – ok

Step-3 – Check 3V ALW _PCH – ok

Step-4 – check 1.8VAlW – not_ok-IC not ok (vcc not present)

Schematics – Follow Voltage Rails (almost available in all schematics)

Power button on keyboard: Near keyboard jack there is a jumper to on the laptop

6- NO Power- NM-D474 REV 1.0 Troubleshoot with Schematics

https://www.youtube.com/watch?v=f0Yubzdbc5U&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=14&t=244s

Step-1 – Check VIN -OK

Step-2 – Check 3V ALW – ok

Step-3 – Check 5V ALW – shorting (not ok)

How to find shorting by injecting voltage and ampier

Remove 5 volt coil leg to check which section (coil in/out)has issue

Note IC side has no shorting but output side has shorting- capacitor has shorting

Schematics – Follow Voltage Rails (almost available in all schematics)

7- NO Power- 6050A2892301-MB-A01Troubleshoot without Schematics

https://www.youtube.com/watch?v=JEpYIYYheOI&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=13

Step-1 – Physical Check

Step 2- Vin

Step 3- charging IC Requirement vcc + ACDet + ACOk-I/o

Step 4- PCH Voltage

Step 5 RTC – not ok

Step 6 LID Switch(3.3v Leg)

Step 7- Always voltage

8- NO Power-NM-D031Troubleshoot with Schematics

https://www.youtube.com/watch?v=1J6pK98LQlI&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=12

Note : jumper pad for MB on/off

Step-1 VIN ok

Step-2 3volt – not ok

Note: 3volt Ic vcc ok and EN1 ok, but EN2 – not ok

Note

EN1 for generating 3 volt

EN2 Generate LDO (for on/off switch and SIO)

Note issue with voltage devider to EN2

8- NO Power-13281-1Troubleshoot with Schematics

https://www.youtube.com/watch?v=VNLkZFLhIh4&list=PLovJQ6y8UZASRVEwmN_p_NPdO7x0_rs8a&index=11

Intel-Power Up Sequence -advanced

AC Mode

Step-1 RTC_AUX_S5- ok

Step-2 RTC_RST# -ok

Step-3 S5_ENABLE (from SIO) -ok

Step-4 3V_5V_EN – ok

Step-5 3V_5V_POK - OK (Power good from 3/5IC)

Step-6 RSMRST# not ok (one of dual mosfet is faulty)