How to find bios by Motherboard or Laptop serial number

Download Bios from authorized site Like Dell HP

Way to extract bios file

- RC On Bios and extract with winrar to find bin file – note if there are no file extension then pull “.bin”

- If file is not extract by win rar(i.e direct .exe) then we can use CMD command.

C:\Users\user\Desktop\Bios Backup\bios file>3442A16.EXE/writeromfile

Note generate two files .rom and .sys

Change .rom to .bin

Sometimes can give error to generate rom file

- Extract by using the software tool (for Dell)

- Lenovo bios gives options Extract only and Install. Then again we get exe.

RC and extract with winrar then we get BIOS.FD

- Temp %temp% by running the exe file in temp folder we get file BIOS1.WPH