EC_RST# ka actual kaam

EC_RST# = SIO ko Reset se bahar nikalna.

Jab EC_RST# LOW hota hai:

- ❌ SIO Reset me hota hai.

- ❌ Firmware execute nahi karta.

Jab EC_RST# HIGH (Release) hota hai:

- ✅ SIO Boot hota hai.

- ✅ EC Firmware chalna shuru karta hai.

- ✅ ACIN, Power Button, Battery, Lid Switch jaise signals monitor karta hai.

✅ Haan, zyadatar motherboards me EC_RST# signal ka pull-up +3.3V ALW/LDO supply se hi hota hai. Main 3.3VS (main regulator) se normally nahi hota.

Kyon?

SIO ko sabse pehle boot hona hota hai.

Us time:

- ✅ +3VALW / +3VL (Always ON LDO) available hoti hai.

- ❌ +3VS (Main 3.3V) abhi generate hi nahi hui hoti.

Isliye EC_RST# ko Always ON supply ki zarurat hoti hai

Voltage

| Condition | EC_RST# |
| --- | --- |
| Reset Active | 0V (LOW) |
| Reset Release | 3.3V (HIGH) |

Jab EC_RST# HIGH (3.3V) ho jata hai, tab SIO firmware execute karna shuru karta hai.

Repair Trick

Agar EC_RST# = 0V hai, to check kare:

- ✅ +3VALW / +3VL present hai?

- ✅ Pull-up resistor open to nahi?

- ✅ Reset IC ya PMIC signal LOW to nahi kar raha?

- ✅ Signal line short to GND nahi hai?

📌 Ek aur important baat: Har motherboard me EC_RST# ka source same nahi hota. Kuch boards me ye Reset IC generate karta hai, kuch me PCH, aur kuch me Power Management Circuit. Lekin pull-up supply almost hamesha +3VALW / +3VL (Always ON 3.3V) hi hoti hai, kyunki SIO ko main power rails ON hone se pehle hi start hona padta hai.

Note:

SUS_CLK → PM_RSMRST# → EC_RST# → Power Button → PM_PWRBTN#

Ye sequence samajh gaya to aap kisi bhi Intel laptop motherboard ki initial power diagnosis aasani se kar paayenge.

Random

Agar PCH EC_RST# control karta ho

Kuch Intel boards me sequence aisa hota hai:

3.3V LDO
      │
      ▼
RTC Domain Ready
      │
      ▼
SUS_CLK
      │
      ▼
PM_RSMRST#
      │
      ▼
PCH releases EC_RST#
      │
      ▼
SIO Boot

Yahan PCH, SIO ko bol raha hai:

"Standby power ready hai, ab tum boot ho jao."

Lekin sab boards me aisa nahi hota

Kuch boards me:

- EC_RST# Reset IC se aata hai.

- Kuch me PMIC se.

- Kuch me PCH se.

- Kuch me RC Reset Circuit se.

Isliye schematic dekhkar hi confirm karna chahiye

Aapke Lenovo NM-A311 board ke context me

Agar aapke schematic me EC_RST# ka source PCH hai, to haan:

PCH
 │
EC_RST#
 │
 ▼
SIO

Ek Important Correction

Aapne likha:

"EC_RST SIO ko stand by rakhne ke liye."

Isko thoda correct karte hain:

❌ Standby rakhne ke liye nahi.

✅ Standby se nikal kar SIO ko firmware execute karne ke liye.

Yani:

- EC_RST# = SIO ko Reset se Release karna.

- SUS_CLK = Standby Clock dena.

- PM_RSMRST# = Standby Domain Ready batana.