- VCC

- ACDET

- REGN 6V  (ic internal work ok)

- ACDRV (to switch on mosfet)

- CMSRC – mosfet on feedback

- AC OK – sio feedback

How to Find VIN Section in Schematics

ACP, ACN – Current measurement of CLR

VCC/DCIN – IC Power Supply

ACIN, ACOK – Output Pins (3.3 volt pull ke throug)

Send msg to SIO that adapter supply CLR par pahuch gya hai(adapter or battery supply is stable)

ACDET – 2.6-3.0V Valid adapter insert hai ya nhi…

ACDRV – mosfet ko on krne keliye.

CMSRC – dono se source se connected, mosfet switch on feedback

REGN ~6V  -VCC/ACDET valid hai to, (IC internally ok hai to)

SRP, SRN – CLR for battery current measurment ke liye.

NOTE

P-Channel Mosfet me ACDRV nhi hota hai kyu ke 25 not required

P channel low  voltage = ON

G=S OFF

Random

Open Drain

Close Drain