Laptop Motherboard Power Concept

Laptop motherboard me voltages ko unke Power State ke hisab se naam diya jata hai.

| Term | Meaning |
| --- | --- |
| ALW (Always ON) | Adapter/Battery lagte hi generate hone wali voltages. |
| VS (Suspend Voltage) | Switched voltages jo S3 aur S0 state me available hoti hain. |
| S0 Voltages (RUN Voltages) | Laptop Fully ON hone ke baad available voltages. |

Note: Repairing field me aksar VS aur S0 voltages ko milakar Secondary Voltages bhi bola jata hai.

Primary Voltages (Always ON)

Ye voltages adapter lagte hi aa jati hain.

+3VALW

Purpose

- EC (SIO)

- BIOS

- Power Button Circuit

- RTC Logic

- Embedded Controller Logic

+5VALW

Purpose

- Charger IC

- Always ON 5V Circuits

- USB Charging (Board dependent)

- Power Management IC

+1.8VALW – Notes Done

Purpose

- 1.8V BIOS Flash

- PCH I/O

- Level Shifter

- Sensors

+1.05VALW – Notes Done

Purpose

- PCH Standby Core

- PCH Internal Logic

Power Button dabane se pehle bhi ye voltages present rehti hain.

Secondary Voltages (Switched Voltages)

Ye voltages Power Button press karne ke baad power sequence ke dauran generate hoti hain.

Generally ye enable hoti hain jab PCH sleep state se bahar aata hai aur SLP_S4# → SLP_S3# → PM_SLP_S0_N sequence complete hota hai.

+3VS

Purpose

- Audio

- LAN

- Camera

- Touchpad

- Sensors

- USB Logic

- Other 3.3V Peripheral Circuits

+5VS

Purpose

- USB 5V

- HDD/SSD

- SATA Devices

- Other 5V Peripheral Circuits

VCCSA (System Agent Voltage)

Purpose

- Memory Controller

- PCIe Controller

- Display Controller

VCCIO

Purpose

- CPU I/O

- CPU ↔ RAM Communication

- CPU ↔ PCH Communication

VCCGT

Purpose

- Integrated Graphics (Intel UHD / Iris GPU)

VCORE

Purpose

- CPU Processing Cores

- Processor Execution

VDDQ

Purpose

- RAM Main Operating Voltage

VCCST

Purpose

- CPU Standby/System Agent related standby rail (Platform dependent)

- CPU ko ON sequence ke liye prepare karta hai.

Repairing Terminology

| Term | Meaning |
| --- | --- |
| Primary Voltages | Adapter lagte hi generate hone wali voltages (Always ON) |
| Secondary Voltages | Power Button ke baad generate hone wali voltages |
| RUN Voltages / S0 Voltages | Laptop Fully ON hone ke baad available voltages |

Technician normally bolte hain:

- ✅ Primary Voltages Check karo → +3VALW, +5VALW, +1.8VALW, +1.05VALW

- ✅ Secondary Voltages Check karo → +3VS, +5VS, VCORE, VCCSA, VCCIO, VDDQ, VCCGT

Repairing Shortcut

| Voltage | Kaam |
| --- | --- |
| +3VALW | EC (SIO), BIOS, Power Button Circuit |
| +5VALW | Always ON 5V Circuits, Charger Related |
| +1.8VALW | BIOS (1.8V), PCH I/O, Level Shifter |
| +1.05VALW | PCH Standby Core |
| +3VS | Audio, LAN, Camera, Touchpad |
| +5VS | USB, HDD/SSD, SATA Devices |
| VCCSA | Memory Controller |
| VCCIO | CPU Data I/O |
| VCCGT | Integrated Graphics |
| VCORE | CPU Core Processing |
| VDDQ | RAM Supply |
| VCCST | CPU Standby/System Agent Rail |