# 1. RTC Power (+3VRTC / VBAT)

### Kaam

RTC section ko chalati hai.

Ye power PCH ke RTC domain ko alive rakhti hai.

Iske functions:

- Date & Time maintain

- BIOS CMOS Settings save

- RTC Clock (32.768KHz) chalana

- PCH ka RTC Logic alive rakhna

# 2. +1.05VALW

### Kaam

Ye PCH Core Standby Voltage hai.

Ye RTC power nahi hai.

Ye PCH ke internal digital logic ko power deti hai.

Functions:

- PCH Standby Logic

- Power Management

- Sleep State Management

- RSMRST# Logic

- SUSCLK Generation (jab RTC domain ready ho)

# Difference

| RTC Power | +1.05VALW |
| --- | --- |
| RTC Section ko power | PCH Core Standby ko power |
| Date & Time | Power Management |
| BIOS CMOS Settings | Sleep Signals |
| RTC Clock | RSMRST#, SLP Signals |
| RTC Domain Alive | PCH Logic Alive |

## 📌 Notes me aise likhiye

### RTC (+3VRTC / VBAT)

- RTC Domain Supply

- Date & Time Maintain

- CMOS/BIOS Settings Save

- RTC Oscillator Support

- PCH RTC Section Alive

### +1.05VALW

- PCH Standby Core Supply

- PCH Internal Logic

- Power Management

- Sleep State Control

- RSMRST#, SUSCLK, Sleep Sequence ke liye PCH ko standby me ready rakhti hai.

### Important Point

+3VRTC aur +1.05VALW dono "standby" se related hain, lekin ek hi cheez nahi hain.

- +3VRTC = RTC Domain Standby Power

- +1.05VALW = PCH Core Standby Power