Main Check Point to Check Laptop Motherboard

- Volt in Confirmation - DC Jack and CLR.

- LDD Confirmation – Battery and On/Off Button.

- SIO Confirmation - SIO se RSMRST and PWRBTN OUT aa rha hai to SIO ki basic Requirement puri hai.

Note – SIO PWRBTN  low and high ho rha hai to SIO Basic Requirement ok hai.

BIO File Currupt hai to bhi sio kaam nahi krega, SIO se PWRBTN low-high nhi hoga.

- SIO and PCH Confirmation - Ram Coil par voltage hai to SIO and PCH ki Basic Requirement puri hai.

- Ram Supply nhi hai to piche ki SIO, PCH check krenge.

- RAM Supply hai to aage ki Regulator  Supply Check krenge- SLP_S3, SLP_S4, SUSP, SYSON ko follow krenge.

Note- Ram Supply ke bad sabi regulator supply on ho jati hai bas Graphic GFX and CPU Core ko chode kr..

Technician View (Har Step par kya check karein)

| Step | Signal / Voltage | Normal |
| --- | --- | --- |
| 1 | VIN | 19V |
| 2 | +3VALW | 3.3V |
| 3 | +5VALW | 5V |
| 4 | EC VCC | 3.3V |
| 5 | RTC Battery | 3V |
| 6 | SUSCLK | 32.768 KHz |
| 7 | EC_RST# / RSMRST# | 3.3V |
| 8 | BIOS SPI Activity | Yes |
| 9 | Power Button | 3.3 → 0V |
| 10 | KBC_PWR_ON | High |
| 11 | SLP_S5# | High |
| 12 | SLP_S4# | High |
| 13 | SLP_S3# | High |
| 14 | VCORE | ~0.8–1.3V |
| 15 | PLTRST# | High |