Check Points to Check Laptop Motherboard

1. VIN Confirmation

Check

- DC Jack Voltage

- Input Fuse

- Charger IC

- CLR (Current Limiting Resistor)

Expected

- 19V (ya adapter voltage)

2. LDO / Always ON Voltages Confirmation

Check

- +3VALW

- +5VALW

- +1.8VALW (Board Dependent)

- +1.05VALW (Board Dependent)

Purpose

Ye voltages EC, BIOS aur PCH ko basic power deti hain.

3. EC (SIO) Basic Confirmation

Check

- RSMRST#

- PWRBTN#

- BIOS SPI Communication

- EC Clock

- EC Reset

Expected

- PWRBTN# press karne par Low → High (ya board design ke hisab se transition)

- RSMRST# High

Note

- Agar BIOS corrupt hai to EC boot nahi karega.

- PWRBTN response bhi nahi milega.

4. EC ↔ PCH Communication Confirmation

Check

- SUSCLK

- RSMRST#

- SLP_S5#

- PCH Reset

Purpose

Ye confirm karta hai ki EC aur PCH communication start ho chuki hai.

5. RAM Power Confirmation

Check

- VDDQ

- VTT

- VPP (DDR4/DDR5)

- RAM Coil Voltage

Note

Agar RAM Voltage nahi hai,

to pehle ye check karein:

- EC

- PCH

- DDR Enable

- Sleep Signals

6. Secondary Voltage Confirmation

RAM voltage ke baad generally ye rails aati hain:

- +3VS

- +5VS

- VCCSA

- VCCIO

- VCCST

- PCH Related Rails

Inke Enable Signals:

- SLP_S5#

- SLP_S4#

- SLP_S3#

- SUSP / SUSP_N

- SYS_ON / PM_SLP_S0_N

7. CPU & Graphics Power

Sabse last me

- VCORE

- VCCGT (Graphics)

- CPU VR Enable

- VR_ON

Ye CPU execute hone se just pehle ya usi sequence me generate hote hain.

Important Notes

✅ BIOS corrupt hone par:

- EC initialize nahi hota.

- PWRBTN response nahi mil sakta.

- Power sequence aage nahi badhegi.

✅ RAM Voltage nahi hai

To piche check karein:

- EC

- BIOS

- PCH

- SLP Signals

- DDR Enable

✅ RAM Voltage aa gayi

To aage check karein:

- +3VS

- +5VS

- VCCSA

- VCCIO

- VCCST

- VCORE

- VCCGT

Ek chhoti correction

Aapne likha:

"Ram Supply ke bad sabi regulator supply on ho jati hai bas Graphic GFX and CPU Core ko chode kr."

Isko thoda technical tarike se likhna better hoga:

"RAM supply generate hone ke baad motherboard ki adhiktar Secondary/System rails enable ho jati hain. CPU Core (VCORE) aur Graphics Core (VCCGT/GFX) ki supplies baad me VR Controller aur CPU ki demand ke according enable hoti hain."

Ye wording Intel aur AMD dono platforms ke liye zyada accurate hai.