Signal ke aage #

# ka matlab hota hai:

Active Low Signal

Yani jab signal LOW (0V) hota hai tab woh Active mana jata hai.

Example

PWRBTN#

PWRBTN#

3.3V ──────────────── Idle

0V  ──┐
      └────────────── Power Button Press

Normally:

PWRBTN# = 3.3V

Power Button dabaya:

PWRBTN# = 0V

Matlab

LOW = Active

RSMRST#

Normally

3.3V

Reset Condition

0V

Release hone ke baad

3.3V

Isliye iska naam

RSMRST#

hai.

SLP_S3#

Laptop ON

3.3V

Sleep Mode

0V

Yani

LOW par Sleep Active.

Agar # nahi hai?

Example

VR_ON

Iska matlab

Ye Active High hai.

0V
↓
OFF

3.3V
↓
ON

Example

VR_ON

Normally

0V

CPU Start

3.3V

Ab PWM Start.

Easy Table

| Signal | Active Level |
| --- | --- |
| PWRBTN# | LOW |
| RSMRST# | LOW |
| SLP_S3# | LOW |
| SLP_S4# | LOW |
| SLP_S5# | LOW |
| SUSCLK | HIGH/Clock Signal |
| VR_ON | HIGH |
| SYSON | HIGH |
| SUSP | HIGH |
| IMVP_EN | HIGH |

Schematic me Kaise Pehchane?

RSMRST# = RSMRST_N = RSMRST_L

Ye teeno ka matlab same hai.

Active Low

VR_ON = SYSON = SUSP

Matlab Active High

Professional Repair Rule ⭐

Motherboard repair me technicians ek shortcut use karte hain:

| Suffix | Meaning |
| --- | --- |
| # | Active Low |
| _N | Active Low |
| _L | Active Low |
| _B | Active Low (kabhi-kabhi) |
| _P | Positive / Active High |
| _H | Active High |

Examples:

PWRBTN#      → Active Low

RSMRST#      → Active Low

SLP_S3#      → Active Low

SLP_S4#      → Active Low

SLP_S5#      → Active Low

VR_ON        → Active High

SYSON        → Active High

SUSP         → Active High

CPU_VR_ON    → Active High